# Testing the characteristic of an ESP32 ADC using DAC

https://how2electronics.com/how-to-use-ads1115-16-bit-adc-module-with-esp32/

Object: to test the linearity, offset and gain errors of an ESP32 ADC 
by comparison with an ADS1115 16 bit ADC

Operation; the ESP32 DAC generates a DC voltage that is applied to the input of both ADC’s

The program generates a series of voltage values from 0 * Vcc/255  to 255 * Vcc/255

For each of those outputs

The program measures the sum of 4 readings from the ESP32 ADC at each of 4 gain settings.

For each of those the reading from the ADS1115 is aggregated, so it’s also a sum of 4 readings.

The program also measures the output from the DAC to check its linearity.  (its very good).

/*

https://randomnerdtutorials.com/esp32-pwm-arduino-ide/

https://how2electronics.com/how-to-use-ads1115-16-bit-adc-module-with-esp32/

ADS1X15 Rob Tillaart installed

*/

/*

experiment using DAC and RC filter  to compare ESP32 ADC with readings from ADS1115

program as esp32 dev module

*/

#include <Adafruit_ADS1X15.h>

Adafruit_ADS1115 ads; /* Use this for the 16-bit version */

//Adafruit_ADS1015 ads;     /* Use this for the 12-bit version */

const int dac1Pin = 25;  // GPIO25, DAC1;

// For a number n (0 - 255) the DAC outputs a voltage n * Vref / 255.

// The default Vref is the 3.3V supply; An external Vref can be used if that pin is exposed on the board

const int vPin = 34;  // Voltage to be measured is connected to GPIO 34 (Analog ADC1_CH6)

int16_t anInRaw = 0;  // raw reading from ESp32 ADC

int16_t ads0;         //raw reading from ads1115

const int tSettle = 50;  //settling time between readings

int32_t readings[260][6];  //260 rows of 6 columns, viz: ADS1115 aggregated; ADC11db; ADC6db; ADC_2_5db; ADC_0dB;  ADS1115 reading DAC

//

void setup() {

for (int i = 0; i < 259; i++) { readings[i][0] = 0; }  //clear this column so ads values can be aggregated

Serial.begin(115200);

delay(2000);  //to allow starting & clearing serial monitor

Serial.println("Comparing raw readings of DAC output comparing ADC1_CH6 and ADS1115");

/*

***** INFO FOR ESP32 ADC *****

analogSetAttenuation(attenuation): sets the input attenuation for all ADC pins. Default is ADC_11db. Accepted values:

ADC_0db: sets no attenuation. ADC can measure up to approximately 800 mV ().

ADC_2_5db: The input voltage of ADC will be attenuated, extending the range of measurement to up to approx. 1100 mV. (1V input = ADC reading of 3722).

ADC_6db: The input voltage of ADC will be attenuated, extending the range of measurement to up to approx. 1350 mV. (1V input = ADC reading of 3033).

ADC_11db: The input voltage of ADC will be attenuated, extending the range of measurement to up to approx. 2600 mV. (1V input = ADC reading of 1575).

analogSetPinAttenuation(pin, attenuation): sets the input attenuation for the specified pin. The default is ADC_11db.

*/

analogSetAttenuation(ADC_11db);  //just confirming ESP32 ADC is set to default

Serial.println("Analog ADC1_CH6 gain set to -11dB range 0 - 2600mV");

/*

***** INFO FOR ADS1115 *****

The ADC input range (or gain) can be changed via the following   functions, but be careful

never to exceed VDD +0.3V max, or to exceed the upper and lower limits if you adjust the input range!

Setting these values incorrectly may destroy your ADC!

ADS1015  ADS1115

-------  -------

ads.setGain(GAIN_TWOTHIRDS);  // 2/3x gain +/- 6.144V  1 bit = 3mV      0.1875mV (default)

ads.setGain(GAIN_ONE);        // 1x gain   +/- 4.096V  1 bit = 2mV      0.125mV

ads.setGain(GAIN_TWO);        // 2x gain   +/- 2.048V  1 bit = 1mV      0.0625mV

ads.setGain(GAIN_FOUR);       // 4x gain   +/- 1.024V  1 bit = 0.5mV    0.03125mV

ads.setGain(GAIN_EIGHT);      // 8x gain   +/- 0.512V  1 bit = 0.25mV   0.015625mV

ads.setGain(GAIN_SIXTEEN);    // 16x gain  +/- 0.256V  1 bit = 0.125mV  0.0078125mV

*/

ads.setGain(GAIN_ONE);                                                       //setting gain for the ads1115:  1x gain   +/- 4.096V  ADS1115 1 bit = 0.125mV

Serial.println("ADS1115 GAIN_ONE ADC Range: +/- 4.096V  1 bit = 0.125mV ");  //this is used for all tests

if (!ads.begin()) {

Serial.println("Failed to initialize ADS.");

while (1)

;

}

}

void loop() {

Serial.println("Testing ESP32 ADC using PWM and ADS1115 \n");

analogSetAttenuation(ADC_11db);  //just confirming ESP32 ADC is set to default

Serial.println("Analog ADC1_CH6 gain set to -11dB");

for (int i = 0; i <= 256; i++) {

dacWrite(dac1Pin, i);  //send value to DAC1

delay(tSettle);

readings[i][0] = ads.readADC_SingleEnded(0);

//take 4 adc readings and aggregate to reduce noise

anInRaw = 0;

for (int j = 0; j < 4; j++) {

anInRaw += analogRead(vPin);

delay(7);

}

readings[i][1] = anInRaw;

}

analogSetAttenuation(ADC_6db);

Serial.println("Analog ADC1_CH6 gain set to -6dB");

for (int i = 0; i <= 256; i++) {

dacWrite(dac1Pin, i);  //send value to DAC1

delay(tSettle);

readings[i][0] += ads.readADC_SingleEnded(0);

anInRaw = 0;

for (int j = 0; j < 4; j++) {

anInRaw += analogRead(vPin);

delay(7);

}

readings[i][2] = anInRaw;

}

analogSetAttenuation(ADC_2_5db);

Serial.println("Analog ADC1_CH6 gain set to -2.5dB");

for (int i = 0; i <= 256; i++) {

dacWrite(dac1Pin, i);  //send value to DAC1

delay(tSettle);

readings[i][0] += ads.readADC_SingleEnded(0);

anInRaw = 0;

for (int j = 0; j < 4; j++) {

anInRaw += analogRead(vPin);

delay(7);

}

readings[i][3] = anInRaw;

}

analogSetAttenuation(ADC_0db);

Serial.println("Analog ADC1_CH6 gain set to 0dB");

for (int i = 0; i <= 256; i++) {

dacWrite(dac1Pin, i);  //send value to DAC1

delay(tSettle);

readings[i][0] += ads.readADC_SingleEnded(0);

anInRaw = 0;

for (int j = 0; j < 4; j++) {

anInRaw += analogRead(vPin);

delay(7);

}

readings[i][4] = anInRaw;

}

//testing DAC1

analogSetAttenuation(ADC_11db);  //just confirming ESP32 ADC is set back to default

Serial.println("Testing DAC1");

for (int i = 0; i <= 256; i++) {

dacWrite(dac1Pin, i);  //send value to DAC1

delay(tSettle);

readings[i][5] += ads.readADC_SingleEnded(1);

}

Serial.println("*** Run complete ***");

printReadings();

//stop when run complete

while (1) {

};

}

void printReadings() {

Serial.println("each value represents the sum of 4 raw readings");

Serial.println("ADS1115 aggregated over 4 runs; ADC11db; ADC6db; ADC_2_5db; ADC_0dB;  ADS1115 reading DAC");

for (int i = 0; i < 259; i++) {

Serial.print(readings[i][0]);

Serial.print(" ,");

Serial.print(readings[i][1]);

Serial.print(" ,");

Serial.print(readings[i][2]);

Serial.print(" ,");

Serial.print(readings[i][3]);

Serial.print(" ,");

Serial.print(readings[i][4]);

Serial.print(" ,");

Serial.print(readings[i][5]);

Serial.print(" ,");

Serial.println("");

}

Serial.println("");

}